Proposed Medical Centre

Orchard House, 28 Church Road, LLanperry

Design of a medical centre at Orchard House, 28 Church Road, Llanperry to involve the conversion and extension of the existing house and re-modelling of the existing grounds to provide parking and landscaped external space.

| # Proposed Medical Centre Orchard House, 28 Church Road, LLanperry ## Introduction Orchard House has been identified for conversion into a new medical centre for Llanperry. The existing house is 110 years old but built in the Victorian style with red brick walls, large bay windows and slate roof. The house sits in private grounds of approximately 0.35 Hectares or 0.9 Acres in a mainly residential area. The site is within easy walking distance of the town centre and has good transport links. The house has been unoccupied for several years and has fallen into a state of disrepair. A rear section of the house was demolished for safety reasons in 2011 and the main body of the house has been boarded and left empty since. In addition to private gardens and drive from the adjacent highway (Church Road) records indicate that the grounds at the rear of the house were used as an apple orchard. This area of the site is also unused and largely overgrown. The new medical centre must accommodate six doctors, three nurses and a receptionist. An extension to the existing house will be required. This will need to be sympathetic to the heritage of the existing building. |  | # Drawings and Illustrations # Click to see larger versions: # Location Plan. # Orchard House |
| --- | --- | --- |
| ## Design Considerations Factors, including requirements and constraints that are likely to tb relevant to the designs for the re-development of this site, as a medical centre. The site Orchard House is located at 28 Church Road, Llanperry. A public highway with wide footpaths that leads to the town centre. The site has a wide vehicular access point from the highway, with reasonable visibility in both directions. It slopes gently up from the highway and is reasonably level from the frontage of the existing house and beyond. The site has a general NW / SE orientation, with the highway towards the SE. The site area is approximately 0.35 Hectares, which is quite large in relation to the accommodation brief provided. Ground conditions at the site comprise heavy clay soils, typical of the area, which will suit standard substructure designs, subject to detailing for frost protection. There are no records of significant archaeological finds in the locality and the site does not suffer from risk of flooding, or excessive noise beyond normal traffic noise associated with a town centre location. All mains services and utilities are available in Church Road. Some mitigation of storm drainage discharge to public sewers should be considered. Planning, The local development plan does not identify specific locations for medical facilities, but it does acknowledge that additional general health care provision will be required should the planned housing expansion in the town be achieved. Initial discussions suggest that the development of the site, as a medical centre, would receive some local support from residents and the conservation benefits in terms of the protection and renovation of the existing house and its environment have been stated by the LPA. |  | # Drawings and Illustrations # Click to see larger version: Site plan as existing |

| ## Design Considerations (continued) Other statutory legislation and government policy Government policy states that health care schemes must respond to their local environment if they are to be adopted by local residents. “This can be achieved by ensuring that the mix of services delivered reflects local needs, and the building demonstrates appropriate levels of urban design and civic presence”. Functional design issues will require compliance with all relevant approved documents of current Building Regulations, including Approved Document M ‘Access to and use of buildings’ to ensure full adherence to the requirements of the Disability Discrimination Act 1995. Secondary design issues will include security, particularly of drugs and medicines (Secure by Design Initiative), fire service access and means of escape, potential for future expansion and the potential conflicts between the needs for privacy and the benefits of providing natural lighting and ventilation. Environmental All buildings providing services to the public have an obligation to incorporate principles of sustainable development. This implies consideration of issues such as, - The reuse of existing buildings and brownfield sites. - Future proofing in terms of flexibility of use and adaptability of layout - SuDS. To retain existing peak run-off rates and annual volumes of run-off. - Health and well-being, including safe pedestrian / cycle links and good internal lighting and comfort conditions. - Minimising energy use and carbon emissions. - Reducing and monitoring water use, including during construction. - Local sourcing of raw and re-claimed materials - Specifying materials that have low environmental impact. - Reducing waste of materials Comply with BREEAM Healthcare methodology and criteria for certification. |  | # Drawings and Illustrations # Click to see larger versions: Building Regulations Sustainability Guide |
| --- | --- | --- |

| ## Design Considerations (continued) Social Consider social accessibility with the aim of reducing travel by private means. Focus should be on design aspects that can impact the levels of satisfaction shown by staff and patients. Note the range of services that are included as community care, in addition to GPs and practice nurses which may include: - health visitors, social workers, and home care advisors. - community midwives. - pharmacists - mental health professionals. Economic Orchard House Medical Practice is an organisation formed by local GPs to arrange for the development and subsequent operation of a new medical centre in Church Road, Llanperry. The development is to be carried out under funding arrangements agreed with the relevant NHS commissioning group which involve the practice mortgaging the development, including the site purchase and all building costs and then being reimbursed their mortgage payments from NHS funds. Initial building costs are to be estimated using UK Government, healthcare premises guides for medium cost general community facilities at the rate of £2400.00 per squ m internal floor area. The quality, functionality and overall performance of the medical centre will need to be designed to accord with the initial cost estimates and any significant variations will need to be brough to the attention of the stakeholders involved in the funding of the project |  | # Drawings and Illustrations # Click to see larger versions: Preliminary cost estimate |
| --- | --- | --- |

| ## Design Considerations (continued) CDM (Construction Design and Management) Regulations. These are the main regulations for managing the health, safety and welfare of construction projects, including all those involved in the design, construction and use of the buildings. The regulations require the identification , elimination, or control of foreseeable risks and place responsibility for planning and managing health and safety in the pre-construction phase on duty holders, namely the Client and principal designer. It will be necessary to complete a risk analysis of the works as part of the overall design and to include the work required in mitigation of the identified risks within the project budgets. The existing building. Plans and elevations of Orchard House are available from the links opposite. The project is to involve the complete refurbishment of the fabric of the house to provide a building envelope complying with current standards. The project is also to involve the re-arrangement of the internal floor layouts to provide the accommodation required. The initial intentions are to include the reception and waiting areas on the ground floor of the original house, with staff facilities on the first floor and with all clinical areas located in new extensions to the rear. |  | # Drawings and Illustrations # Click to see larger versions: # Front Elevation Other Elevations Floor Plans |
| --- | --- | --- |

| ## Initial Project Brief A brief for the Client to meet the business, development and user needs taking account of the stages involved in the design process. Parameters End Users GPs. Functional clinical accommodation with adequate space and privacy for examinations, based on NHS guidelines for consulting rooms. Nurses. As above with suggestion to retain room areas and layouts for flexibility and standardisation of fittings. Other staff. Safe and comfortable working space in reception area, with more private facilities for rest periods and toilet accommodation separate from public area. Patients. Comfortable / relaxing waiting area with easy access to parking space, reception, and consulting rooms. Aesthetic requirements Retain character of existing house. Select materials for new build to match the existing. Welcoming entrance and light, airy waiting area with visual connection to external areas. Implications arising from physical environment. Access from Church Road with parking to guidelines based on number of consulting rooms and projected staffing level. Utilise existing house on both levels, maintaining frontage, with extensions to side/rear. |  | # Drawings and Illustrations # Click to see larger versions: # Spatial arrangement. # Sketch of consulting room. |
| --- | --- | --- |

| Parameters (continued). Statutory constraints No specific planning constraints other than standard planning conditions regarding commencement and approval of external finishes. Compliance with Building Regulations and NHS requirements for sustainability and BREEAM certification. Project outcomes Conservation of existing building and provision of community medical centre that meets the requirements of all stakeholders in terms of facilities, budget, and community needs. Possible design ideas and specifications Refer to initial sketch layout plan. Specification for new build to include single storey accommodation around courtyard to rear of existing house, with pitched roof, generally to match the existing. Possible use of SIPs for external walls and internal partitions to provide high levels of thermal insulation and acoustic privacy. Consider total site area in relation to the requirements of the medical centre. Suggest possible development of surplus land as housing as means to reducing borrowing requirements. Include any proposals for housing as part of planning application for the medical centre. Pre-construction activities regarding the existing building. Carry out structural and condition survey and prepare schedule of repairs and renovations. Develop specifications for improved fire resistance / compartmentation of first floor areas and insulation of building envelope. |  | # Drawings and Illustrations # Click to see larger versions: Initial sketch layout for medical centre, with housing to rear. # Sketch of proposed courtyard to rear, with access from waiting room. |
| --- | --- | --- |

| ## Project Designs Project design sketches and drawings in 2D and 3D to explore and explain initial ideas. Building plans Sketch plan illustrating initial ideas for the layout of a single storey extension to provide consulting / nurses rooms and ancillary accommodation. Sketch plans illustrating revised layouts for the exiting house, with reception and waiting spaces on ground floor, and staff / office facilities on the first floor. Site plan The site plan illustrates the existing house complete with the proposed extension to the rear. The areas to the front and side of the building have been developed to provide access and parking with 8 standard and 2 disabled spaces to the front and a further 20 public parking spaces to the side. The layout includes ramped pedestrian access to the main entrance and turning spaces that will accommodate large vehicles such as an ambulance.. uuuuuuuuuuuu mmmmmmmmmm Building elevations Mmmmmm Technical notes Mmmmmmmmm Component and material specifications Mmmmmmmmm |  | # Click to see larger versions: Extension: Sketch layout Existing house. As proposed Proposed reception area. |
| --- | --- | --- |

| ## Project Designs Project design sketches and drawings in 2D and 3D to explore and explain initial ideas. Building elevations Renovation of existing house to include stone cleaning and repair. New entrance with feature canopy, replacement widows, re-roofing with new rainwater goods, facias, and soffits. Cleaning of existing facing brickwork to be subject of further on-site investigation to establish extent of repairs possible without causing undue damage to existing materials. Exiting chimneys to be retained as a design feature, with possibility of including open fire or other flued heating appliance in staff restroom. The Extension to the rear of the existing house is to be built using facing bricks, reconstituted stone sills and heads, and slate roof all to match the existing, with powder coated aluminium framed windows to match existing proportions. Corridor walls to courtyard to be fully glazed as shown in the courtyard elevation opposite, with possibility to utilise solar gains and heat exchangers to supplement domestic hot water systems. # Click to see larger versions: Typical Section Site Plan showing proposed service routes and utilities. Technical, Component and Material specifications See notes on elevations for descriptions of external finishes. The design considerations for community buildings of this nature emphasise the importance of future proofing in terms of flexibility of use and adaptability of layout. The open plan nature of the proposed public / waiting areas provides access for future extension to the side of the existing house should a larger meeting space or clinic room be required. (see building plans). The design of the extension includes some additional structural elements, as indicated on the typical section drawing, to enable intermediate partitions to be removed to provide larger spaces and to facilitate vertical extension assuming a precast concrete floor with cantilevered section over the corridor and designing elements such as roof trusses for re-use. The stated design considerations also list the following: - SuDS. To retain existing peak run-off rates and annual volumes of run-off. - Minimising energy use and carbon emissions. - Local sourcing of raw and re-claimed materials - SuDS proposals include a central pond to receive and store storm water from roofs and hard areas for use I garden watering etc. The pond will require a top up water supply and an overflow to drain to avoid flooding. The onsite system with pond should alleviate run-off from a peak flow generated by a 6hour, 1 in 30 year storm. - High insulation values and limited use of gas fueled space heating, together with a controlled ventilation system, with heat re-claim, supplemental use of captured soler gains and PV generation will all contribute towards achieving the required BREEAM certification. - Use of wool based insulants, FSC certified timber products, Welsh slates and locally produced cements, concrete products and facing bricks will accord with sustainability requirements, together with use of re-cycled aggregates as base materials for external roadways and parking areas. |  | # Drawings and Illustrations # Click to see larger versions: Front elevation, as proposed Rear Elevations as proposed |
| --- | --- | --- |

| ## 2D and 3D Modelling Design models in 2D and 3D to illustrate design ideas in realistic detail. Building plans The building plan was developed as a 2D model, with layers for walls, windows, and doors, plus text for room labels. An additional layer was added to show fixtures, fittings, and furniture. The final layer adds all necessary dimensions. The drawings were prepared using CAD software as shown. |  | # Click to see larger versions: Basic plan Plan with Furniture and Fittings. Plan with Furniture and Fittings. |
| --- | --- | --- |

| ## 2D and 3D Modelling Site plan The site plan has been developed using graphics software with a second layer to illustrate a proposal to develop the area of the site to the rear, which is not needed at this stage, as housing. This limits the potential for the future extension of the medical centre and may not be thought appropriate at this stage but including the houses as part of the planning application for the medical centre allows the option to be considered, particularly if it helps with funding. Building envelope A 3D version of the model was created by turning the basic 2D model (with all layers off, except the wall layout) into an isometric view and then extruding the walls vertically by the ground floor storey height to form a wire frame. The wire frame was then extended by raising the walls to form the first floor of the existing house, adding the outline geometry of the roof slopes and outlines of the external windows. The wire frame was then shaded and rotated 3600 to select a view for presentation. The perspective view of the model was then imported into an image to provide a realistic setting. |  | # Click to see larger versions: Proposed Site Plan Wire Frame Perspective View |
| --- | --- | --- |

| ## Evaluation of design and construction proposals Review of - The building design, - Proposed construction methods, - Sustainability measures - Site layout. Building requirements (primary and secondary) The primary requirements of the design brief have been achieved in that the proposed building provides the level and type of accommodation required for the GPs and nurses. The treatment rooms are based on standardised 16 m2 layouts to provide space for consultations, IT facilities and treatment. The rooms are flexible in that adjacent rooms cand be combined by removal of partitions to provide larger rooms if required. The solid sound proofing partitions could be replaced with lighter weight removable screens if this was thought to be helpful. The secondary requirements in terms of providing a welcoming entrance and relaxing waiting area for patients has been achieved in the utilisation of the existing house with large bay window to the front and extended glazed opening that allows access to the rear courtyard to create a light and airy space that could be further enhanced by the selection of colourful and comfortable furnishings. The main entrance is given due emphasis by the addition of a modern glazed canopy. The secondary requirements in terms of providing a suitable and safe working environment for support staff has also been achieved in the utilisation of the existing house, with an accessible reception area at ground floor allowing supervision of patients, movements, and with secure office, restroom and kitchen facilities on the first floor, complete with passenger lift access to enable employment of staff with disabilities and with opportunities to provide escape stairs to the parking area from the restroom, and / or emergency exit from the lift lobby on the ground floor. |  |
| --- | --- |

| ## Evaluation of design and construction proposals (cont.) Conservation of the existing building The design acknowledges the importance of the existing house, placing it centrally both in terms of functionality and aesthetics. Several features of the house have been replicated in the extension and materials are to be specified to match. The same system is to be used for the replacement windows in the existing as for the new build and proportions of the openings are similar. The existing chimneys, which are a notable feature of the existing deign are to be repaired, retained and re-used to provide flues for feature heating appliances and for vertical circulation of services. The stonework is to be repaired and cleaned and the existing slates are to be salvaged and reversed for re-use to complete at least 50% of the pitches. Lead flashings are to be salvaged and replaced with new, installed to traditional details. Substructures and superstructures The existing house shows no signs of subsidence and therefore underpinning or similar repairs to the original substructures are not anticipated, although the existing suspended timber ground floor is to be removed and replaced with new concrete slabs, which are to be ground bearing where fill depths allow. The foundations and floor slab of the extension are to be constructed at similar depths to limit differential settlement, and to take account of possible effects of frost heave in the clay sub soil. |  |
| --- | --- |

| ## Evaluation of design and construction proposals (cont.) Proposals for sustainability In addition to the salvage / re-use of materials from the existing house and the sourcing of new materials locally the designs for the medical centre include measures for the capture of passive solar gains, use of photovoltaic panels to supplement electricity, and a recirculating ventilation system with heat exchange facilities to limit heat loss. These measures together with high levels of insulation throughout the new extension and the introduction of insulation below the ground floor, to the walls, and in the roof space of the existing house will help minimise energy use. Water and wastewater systems The water systems required are not extensive and proposals for water conservation are minimal, although the SuDS approach of collecting all storm water for use for garden watering should reduce water use and avoid peak flows from entering the public system. Landscape and site design Proposals for the hard external areas are to include brick paving to roadways and parking spaces, with contrasting stone coloured concrete paving to footpaths. The paving to be selected to match colours of the existing house. Landscaped areas are to include a grassed area to complete the frontage to church road, with planting generally concentrated towards the site boundaries and as screening to the main parking areas. |  |
| --- | --- |