The first thing to do is to read the brief and to identify the requirements for the non-examined assessment (NEA).  Each year the brief for the NEA will change but the way to tackle it will be the same.

Example:

The Brief

A Welsh town has experienced significant population growth over the last ten years and projections suggest the trend is set to continue. As the population continues to rise, there is an increased demand for access to General Practitioners (GPs). There is insufficient space for a new-build medical centre, but a derelict property has been identified for conversion into a new medical centre for the town. The existing building is 110 years old and constructed mainly of brick. It has two storeys and a pitched roof. It has a square footprint with two equally sized rooms downstairs and four equally sized rooms upstairs. It sits in private grounds of approximately one acre in a mainly residential area. The new medical centre must accommodate six doctors, three nurses and a receptionist. The existing building will have to be extended to address users’ needs. The extension will need to be sympathetic to the heritage of the existing building.

You have been contracted to design and plan the conversion and extension of the existing building into suitable accommodation for the medical practice.

Activity 1: Requirements of the brief

Each NEA brief will set the scene for the task to be carried out.  It is important to identify the client’s requirements to allow you to plan your work for the NEA.

End users: all stakeholders who will access the building including those who work there and all visitors.

Accommodation: the requirements for the building including size, number and type of rooms and other facilities

Other factors: access to the building, other facilities required on site such as parking, constraints on the project such as the need to demolish or convert existing buildings.

End users

Accommodation

Other factors.

Enter a list of the client’s requirements for the proposed development taken from the brief for your NEA.

[INPUT: Textbox OUTPUT:  Form…….]

Activity 2: Location

Every NEA will need a location identified for the construction or conversion of a building.  Use the information from the brief to help you locate a suitable site.

Nature of site

Example:

Example

Enter a description of the site you have chosen for your building from the NEA requirements.  This should include a location plan showing the site in relation to services such as roads and other properties.

[INPUT: Textbox OUTPUT:  Form……]

Nature of site

Residential area: a district where people live, where housing dominates, as opposed to industrial and commercial areas.

Size of the site: approximately one acre

Existing building: 110 years old, in poor condition.  It has a square footprint, with two rooms on the ground floor and four rooms on the first floor.

Other requirements:

- suitable access to the site for vehicles and pedestrians.

- Sufficient space for parking and access for emergency vehicles.

- Ground conditions at the site.

- Availability of mains services and utilities.

Activity 3 – Planning and statutory regulations

Planning

Statutory regulations

Example

Enter a description of the planning and statutory regulations that will affect the design of your building for the NEA.

[INPUT: Textbox OUTPUT:  Form……]

You will need to identify the statutory requirements that will affect the design and construction of a building including:

- National Building Regulations and Building Standards

  - What qualifies as building work

  - The notification procedures that must be followed

  - The requirements for building design and construction (Approved documents A – R).

In particular, Approved Document M that sets ‘Access to and use of buildings’ to ensure that any design complies with the Disability Discrimination Act.

You need to consider the parameters of the planning process including:

- Compliance with the Local Plan and other government policies

- Impact on the local area

- Effect on trees and wildlife and the impact on any Site of Special Scientific Interest (SSSI) or Area of Outstanding Natural Beauty (AONB)

- Highway matters, traffic access, visibility and parking scope of existing local facilities and local housing stock

- Potential loss of light or privacy to neighbouring properties

- Suitability for the local area, issues of conservation and listed buildings

- Over development

- Provision or retention of employment

- Issues concerning drainage and surface water runoff.

Activity 4 – Environmental and social issues

Environmental issues

Social issues

Example

Enter a description of the environmental and social requirements and constraints that will affect the design of your building for the NEA.

[INPUT: Textbox OUTPUT:  Form……]

You will need to identify social requirements that could constrain the design process including:

- The needs of stakeholders including the local community

- The needs of the client’s workforce

- Limitation on the development arising from public opposition and/or media pressure.

You will need to identify environmental parameters that could constrain the design process including:

- Use of sustainable or hazardous materials

- Energy consumption and carbon emissions

- Air, water or ground pollution

- Waste and water management

- Noise, vibration and dust

- Traffic and transport

- Preservation of ecology

- Resilience to climate change

- Design for deconstruction and disposal

Activity 5 – Economic requirements and constraints.

Economic constraints

Example

Consider the costings and economic constraints that will affect the design of your building for the NEA.

[INPUT: Textbox OUTPUT:  Form……]

You will need to identify possible impacts of economic constraints on the project budget and the allocation of resources in terms of:

- Quality

- Safety

- Functionality

- Performance

- Quality

- Safety

- Functionality

- Performance

Activity 6 – Construction (Design and Management) Regulations

Construction (Design and Management) Regulations are the main set of regulations for managing health and safety for construction projects.

CDM regulations apply to all stages of a construction project including the design phase.

Responsibility for planning, managing, monitoring and coordinating health and safety is the duty of:

- Client

- Principal designer

- Principal contractor

- Workers.

Example

Consider the health and safety constraints that will affect the design of your building for the NEA.

[INPUT: Textbox OUTPUT:  Form……]