12. Building services engineering (BSE) systems

# Worksheet 1: Air conditioning components (tutor)

Complete the table below by matching the description to the AC component listed:

cooling coil, recirculation duct, preheater battery, supply fan, reheater battery

| AC component | Description |
| --- | --- |
| Preheater battery | In the winter cycle fresh air is heated to 7OC preventing the filters from freezing. |
| Supply fan | The duty of this fan is slightly greater than that of the return air fan which gives a positive air pressure in the room. |
| Recirculation duct | Up to 25% of room air can be mixed with the fresh air, utilising already conditioned air. |
| Reheater battery | The air is heated to the supply air temperature (34ºC) for comfort and to compensate for heat loss throughout the building. |
| Cooling coil | Cooled by either chilled water or refrigerant gas. Moisture is removed from the air by dehumidification. |