12. Building services engineering (BSE) systems

# Worksheet 11: Mechanical principles (tutor)

- Briefly describe how a centrifugal fan works.

When the fan impeller is rotated by the fan motor, air is induced because the impeller creates a low negative pressure in the chamber. The air is slightly compressed and forced out of the outlet into the ductwork.

- What type of pump is used to create high enough pressures to lift water in high-rise buildings?

Multi-stage centrifugal pump.

- Sketch a drawing of a ground source heat pump heating a house in cold weather. Your diagram should include the following labels:

ground heat exchanger, direction of the hot and cold water, where heat extraction occurs, heat pump unit, heat distribution system