12. Building services engineering (BSE) systems

# Worksheet 14: Cables and equipment in older electrical installations (tutor)

- Name two different types of cable sheathing which are no longer used.

Lead sheathed cable and vulcanised rubber cables.

- Why is lead sheathed cable no longer used?

Lead is a soft metal which reacts to changes in temperature and will expand and contract with any changes. This will result in the lead becoming brittle, causing it to crack and allowing moisture to enter the sheathing.

- You are involved in refurbishing an old heritage property. What types of fuses might you find in the consumer unit?

Rewire fuses (now called semi-enclosed, BS3036).

- You attend a house with a plastic clad consumer unit. Explain to the customer why this is now considered substandard.

Plastic consumer units can cause electric fires due to loose connections or short circuits. They must now be replaced with metal-clad fire-rated consumer units (BS7671).