12. Building services engineering (BSE) systems

# Worksheet 5: Y- and S-plan systems (tutor)

- Note down the key elements of a Y-plan and an S-plan fully pumped heating system.

| Y-plan | S-plan |
| --- | --- |
| The key element of this system is the three-port mid-positional motorised valve which splits the flow of water from the boiler to the hot water storage cylinder and the heating system as and when there is a call for heat. | The key elements of this system are the two two-port motorised valves which control the flow of water from the boiler to the hot water storage cylinder and the heating system as and when there is a call for heat. |

- Which type of system is shown below – a Y- or an S-plan?

This is a Y-plan system.

- Answer the following questions about the magnetic system filter.

- What is the purpose of a magnetic system filter?

- Where in the system should it be positioned?

- The filter chamber can be removed and cleaned without draining the system. True or false?

- It is designed to combat the damaging effects of system debris and pollutants.

- It is installed on the return pipe below the boiler.

- True.