# 13: Maintenance principles

# Worksheet 2: Research activity (tutor)

Research typical maintenance activities for one of the BSE systems below:

- fire detection and smoke alarm systems

- air conditioning system

- intruder alarm system.

Write a maintenance plan, listing how regularly the chosen system should be maintained and what tasks need to be carried out.

Maintenance plan for: Air conditioning system

Timeframe

Typically conducted every 12 months.

Typical tasks

- Check the unit’s electrical wiring to ensure it is safe to use.

- Clean vents and grilles.

- Clean and inspect motors.

- Check the condenser coil and clean as required.

- Inspect and adjust belts and pulleys.

- Check any condensate connection is installed correctly.

- Use a leak detector device to test for any leaks from the refrigerant.

- Check the operation of any components, appliances and safety controls.

- Record the temperature and pressure.

Learners will have different answers: here is a sample answer.