3. Construction design principles

# Worksheet 4: Brownfield and greenfield sites research task (tutor)

Research the differences between brownfield and greenfield sites. Complete a table for each type of site, listing the advantages and disadvantages for development.

Brownfield sites

| Advantages | Disadvantages |
| --- | --- |
| Planning permissions are easier to obtain as local authorities are trying to improve run down and derelict land within and surrounding the existing urban area. | Contaminated ground is expensive to clear. |
| Can stimulate local regeneration, shops, business, etc. | Can be in less desirable areas of towns and cities, making properties less attractive to homebuyers |
| Services and infrastructure are normally already in place. |  |
| Developing brownfield sites reduces the pressure on housing that would otherwise lead to cities/towns expanding into greenfield sites. |  |
| Clearing brownfield sites can reduce contamination and pollution for the longer-term sustainability of the land. |  |
| Allows housing to be built closer to places of work, giving employment opportunities for residents and reducing commuter traffic. |  |

Greenfield sites

| Advantages | Disadvantages |
| --- | --- |
| Land is normally free from contaminates. | Government and local authorities have increased restrictions on the development of greenfield land to protect wildlife and the countryside. |
| Homes often command a premium price as they are in desirable semi-rural or rural locations. | Planning permissions are often more difficult to obtain. |
| More space is available for gardens for homes, or for larger commercial developments such as business parks. | Services and infrastructure will need to be provided. |
|  | Increased pollution is likely due to urban sprawl. |