| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 3: Off‑Cycle Defrost: Feasibility, Timing & Energy

Scenario

A medium‑temp case holds +4 °C air and runs SST ≈ −6 °C. Off‑cycle defrost is scheduled 6×/day. During off‑cycle, the compressor stops, and the fans run.

Assume

(a) Coil + frost equivalent heat capacity Ceq = 14 kJ/K;

(b) Average air‑to‑coil UA = 65 W/K;

(c) Coil temp at start of defrost = −8 °C;

(d) Melt begins at 0 °C.

Treat the coil as a lumped mass exchanging with room air at +4 °C.

1) Using T(t) = Tr − (Tr − T0)·exp(−(UA/Ceq)·t), calculate the time to reach 0 °C (start of melting). (Convert UA to kJ/s/K.)

2) If mfrost = 0.8 kg, estimate additional time to melt by assuming sensible heat from room air only at an average ΔT of 3 K and the same UA. (Latent of fusion = 334 kJ/kg.)

3) Check feasibility: if total exceeds 10 min, propose a revised frequency (defrosts/day) or off‑cycle duration to fully clear frost without heaters.

Extension task

Suggest a demand‑based initiation strategy using a low‑pressure cut‑out or fan pressure‑differential, with one advantage and one risk.