| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 2 K1.2 Fluids – Disposal of recovered fluids (oil, ammonia water, secondary refrigerants)

Questions

Use density of water = 1.0 kg/L unless told otherwise.

1) Waste oil: 135 L used compressor oil (assume hazardous).
   a) 25 L UN drums required?
   b) Charges £18 per drum uplift + £0.22/L treatment → total cost?
   c) Two duty‑of‑care checks and which paperwork applies?

2) Ammonia‑contaminated water: 230 L.
   a) 30 L UN drums required?
   b) Mass estimate (kg)?
   c) Why not drain without consent?

3) Secondary refrigerant (glycol): 420 L to replace; IBCs can be filled to max 950 L.
   a) IBCs needed?
   b) Treatment charge £0.38/L → cost?

Extension

Packaging choice for 480 L:

Option A (30 L drums £5/drum + £0.35/L) OR

Option B (IBC hire £40 + £0.30/L).

Which is cheaper and by how much?

Tutor guidance

Answers

1. Waste oil: 135 L used compressor oil (assume hazardous).

a) Number of 25 L UN drums required

135 L25 L/drum=5.4⇒6 drums 
b) Charges

Uplift: 6 drums×£18=£108

Treatment: 135 L×£0.22=£29.70

Total cost: £108 + £29.70 = £137.70

c) Carrier reg + permitted destination; HWCN; 3‑year retention.

2. Ammonia‑contaminated water: 230 L.

a) 30 L UN drums required?

23030=7.67⇒8 drums

b) Mass estimate (kg)?

Density of water = 1.0 kg/L

230 L≈230 kg

(Ammonia traces don’t change mass much.)

c) Why not drain without consent?

Harmful to aquatic life; trade effluent consent needed.

3. Secondary refrigerant (glycol): 420 L to replace; IBCs can be filled to max 950 L.
a) IBCs needed?

IBCs needed (max 950 L)

420950<1⇒1 IBC

   b) Treatment charge £0.38/L → cost?

420 L×£0.38=£159.60​

Extension

Packaging choice for 480 L:

Option A — 30 L drums

480 L / 30 L = 16 drums

- Drum cost: 16×£5=£80

- Treatment cost: 480×£0.35=£168

Total A = £248

Option B — IBC

- Hire: £40

- Treatment: 480×£0.30=£144

Total B = £184

Cheaper option

£248-£184=£64

Option B is cheaper by £64