| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 1: Air Conditioning Systems

K1.1 The function and operation of air conditioning systems

1. Which best describes a direct expansion (DX) system?

A. Uses chilled water to cool air
B. Uses refrigerant directly in the evaporator coil to cool air
C. Uses a ground loop to extract heat
D. Uses a cooling tower for heat rejection

2. Which component in a DX system absorbs heat from indoor air?

A. Condenser
B. Compressor
C. Evaporator
D. Expansion valve

3. What is the main advantage of a split DX system compared to a packaged (unitary) unit?

A. Lower installation cost
B. Better zoning and reduced noise indoors
C. No outdoor unit required
D. Uses chilled water instead of refrigerant

4. Which statement correctly compares an AHU and an FCU?

A. Both provide outdoor air ventilation
B. AHUs are small and installed in individual rooms
C. FCUs provide local conditioning, AHUs provide ventilation and filtration
D. FCUs can only heat, not cool

5. What principle does a chilled beam mainly use to cool a space?

A. Conduction
B. Natural or induced convection
C. Radiation only
D. Evaporation

6. What does a heat pump do in heating mode?

A. Produces heat from combustion
B. Moves heat from outside to inside
C. Cools water using chilled beams
D. Transfers heat into the ground

7. What is a key difference between air-source and ground-source heat pumps?

A. Air-source units are installed underground
B. Ground-source units use stable ground temperatures
C. Ground-source units cannot heat buildings
D. Air-source units require cooling towers

8. What distinguishes VRF/VRV systems from traditional DX systems?

A. They use water as the main heat transfer medium
B. They vary refrigerant flow to meet zone demand
C. They cannot provide simultaneous heating and cooling
D. They have no outdoor units

9. How can VRF/VRV systems provide simultaneous heating and cooling?

A. By installing two completely separate systems
B. By using heat-recovery and branch selector technology
C. By running at maximum compressor capacity
D. By using only outdoor air

10. What does a water chiller do in an HVAC system?

A. Heats water using refrigerant
B. Produces chilled water by removing heat from the water loop
C. Supplies ventilation air
D. Expels refrigerant to the atmosphere

11. Which is a key difference between air-cooled and water-cooled chillers?

A. Water-cooled chillers use refrigerant, air-cooled do not
B. Air-cooled chillers reject heat to outdoor air, water-cooled use cooling towers
C. Water-cooled chillers require no plant room
D. Air-cooled chillers are always more efficient

12. What happens to system equipment under low cooling loads?

A. Compressors run at maximum capacity
B. Pumps and compressors slow down to save energy
C. Fans turn off completely
D. Chillers shut down immediately