| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 2: p–h Chart Cycle Labelling

K1.10 Principles of operation of heat pumps

What you will do

Interpret the axes of a pressure–enthalpy (p–h) chart.

Label the four processes on a heat pump cycle and link each to a component.

Identify where heat is absorbed and rejected and where work is added.

Resources

Your class p–h chart handout or the p–h chart slide in the lesson.

Pen/pencil + highlighter.

Instructions (15–20 minutes)

- Work in pairs.

- Use the p–h chart shown in the lesson and follow the cycle direction 1 → 2 → 3 → 4 → 1.

- Complete the tasks below and write answers clearly.

Tasks

Label the axes: what is on the x-axis? What is on the y-axis?

Mark points 1, 2, 3, 4 on the cycle and write what component sits between each pair:

Identify which process is approximately constant enthalpy (h ≈ constant).

Shade/underline the section where the refrigerant absorbs heat from the source.

Shade/underline the section where the refrigerant delivers heat to the building.

Circle the section that represents compressor work input.

Cycle table (fill in)

| Process | Component | What happens to pressure? | What happens to enthalpy? | Heat/work interaction |
| --- | --- | --- | --- | --- |
| 1 → 2 |  |  |  |  |
| 2 → 3 |  |  |  |  |
| 3 → 4 |  |  |  |  |
| 4 → 1 |  |  |  |  |

Maintenance link (write one each)

For each component, write ONE maintenance check that helps performance (e.g., coil cleanliness, airflow, superheat).

| Component | Maintenance check |
| --- | --- |
| Evaporator |  |
| Compressor |  |
| Condenser |  |
| Expansion device |  |