| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 4: Compare Efficiency & Running Costs

Scenario

A home needs 12,000 kWh of heat over a heating season.

Assumptions

Gas boiler seasonal efficiency = 90% (0.90)

Air source heat pump (ASHP) seasonal COP = 3.0

Ground source heat pump (GSHP) seasonal COP = 4.0

Electric resistance heating = 100% (1.00)

Part A: Energy input required

Use: Energy input = Heat demand / efficiency (or / COP)

| System | Efficiency / COP | Energy input needed (kWh) |
| --- | --- | --- |
| Gas boiler | 0.90 |  |
| ASHP | 3.0 |  |
| GSHP | 4.0 |  |
| Electric resistance | 1.00 |  |

Show your working below:

Part B: Running cost (use your local tariffs)

Write the unit prices you will use (per kWh):

| Energy type | Unit price (p/kWh or £/kWh) |
| --- | --- |
| Electricity |  |
| Gas |  |

Calculate the seasonal running cost for each system (use your energy inputs from Part A):

| System | Energy input (kWh) | Estimated cost (£) |
| --- | --- | --- |
| Gas boiler |  |  |
| ASHP |  |  |
| GSHP |  |  |
| Electric resistance |  |  |

Part C: Discussion (answer in sentences)

1) Which system uses the least input energy? Why?

2) Why might the cheapest-to-run system be different from the lowest-energy system?

3) Give two maintenance or design actions that help a heat pump maintain a high COP.

4) When might a GSHP be preferred over an ASHP (site and performance reasons)?

Extension (challenge)

If an ASHP COP falls from 3.0 to 2.2 during cold weather, what happens to the electricity input needed for 12,000 kWh heat?