| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 5: Maintenance & Fault Symptoms (Diagnosis)

Aim

Identify likely causes of common performance symptoms.

Choose safe first checks a technician would make.

Explain how faults can change pressures/temperatures and reduce COP.

Instructions (15–20 minutes)

Work in pairs.

For each symptom, choose the most likely cause(s).

Write TWO safe first checks you would carry out (e.g., visual inspection, airflow, filter condition, temperature readings).

Do not suggest unsafe actions.

Symptom → Cause → Checks

| Symptom | Most likely cause(s) | Two safe first checks | How COP/cycle might change (brief) |
| --- | --- | --- | --- |
| Low heat output + low suction pressure + high superheat |  |  |  |
| High discharge pressure + poor heat output |  |  |  |
| Outdoor unit icing + frequent defrost |  |  |  |
| High power draw + noisy compressor |  |  |  |
| Short cycling (on/off frequently) + comfort complaints |  |  |  |

Plenary: Explain in your own words

1) Why does reduced airflow across a coil reduce heat pump efficiency?

2) Why is correct flow temperature (emitters/controls) important for COP?

3) One key thing I learned today about p–h charts is: ____________________

Exit ticket (1 minute)

Write one question you still have about heat pumps, COP, or p–h charts: