| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 6: Heat Pump Basics (Starter)

K1.10 Heat pumps | System maintenance

Use Q = m c ΔT, Q = m L, and Power = Q/Δt. Take c_p(water)=4.186 kJ/kg·K, c_p(air)=1.005 kJ/kg·K.

- Sensible cooling

How much energy (kJ) is removed to cool 2.5 kg of water from 30 °C to 5 °C?

- Latent + sensible

How much energy to melt 1.2 kg of ice at 0 °C and then heat the water to 20 °C? (L_f(ice)=334 kJ/kg)

- Energy over time

An AC removes 8.5 kW for 2 hours. How much energy is removed in kWh and kJ?

- Coil sensible load from airflow

Airflow = 0.60 m³/s, ρ=1.2 kg/m³, cool from 28 °C to 16 °C. Find sensible load (kW).

- Refrigerant mass flow

If coil load is 12 kW and refrigerant enthalpy drop is 35 kJ/kg, find mass flow (kg/s).

Extension

1) An AC delivers 7.5 kW of cooling with COP = 3.2 for 3 hours. Find electrical energy (kWh) and cost at £0.28/kWh.

2) Heat 10 kg of glycol (c_p ≈ 3.6 kJ/kg·K) from 10 °C to 65 °C in 25 minutes. Find energy (kJ) and average power (kW).

3) For a 12 kW evaporator, find refrigerant mass flow (kg/s) if Δh is (i) 28 kJ/kg and (ii) 42 kJ/kg. Comment on how Δh affects ṁ.