| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 5: Fault-Finding from Commissioning Data (Evidence-based decisions)

Instructions

For each mini-case, use the pattern of readings to select likely causes and next checks.

Important: these are learning scenarios. Real systems must be checked against manufacturer data and safe procedures.

A) Mini-cases

| Mini-case | Symptoms / readings (steady-state) | Most likely issue (choose 1) | First THREE checks you would do |
| --- | --- | --- | --- |
| A | SH high, SC low; room not reaching set point; suction pressure low | Undercharge / leak / restriction / airflow | 1) 2) 3) |
| B | SH low, SC high; head pressure high; amps high | Overcharge / condenser airflow / restriction / sensor error | 1) 2) 3) |
| C | Air ΔT small; pressures near expected; room reaches set point quickly; inverter ramps down | Low load / airflow too high / normal operation / charge issue | 1) 2) 3) |
| D | Suction pressure low; coil icing; SH low; filters visibly dirty | Low airflow / undercharge / metering fault / control error | 1) 2) 3) |

B) Reflection

1. Why is “add refrigerant” not a safe first response when performance is poor?

2. Explain why you should make only ONE change at a time during commissioning.

3. Give TWO examples of measurement error that could lead to a wrong diagnosis.

Practical task (role-play)

One learner is the engineer, one is the customer. Explain what measurements were taken and what the results mean in simple terms.