| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 2: Measure Fresh Air and Ventilation Effectiveness

K1.2 Air conditioning and ventilation in a modern economy

Scenario:

Measure ventilation performance and identify stale-air removal.

1. Airflow measurement at a ventilation grille

Use an anemometer

Record air velocity (m/s)

Measure size of the grille (m²)

Calculate airflow volume (m³/h)

Use the formula:

Airflow (m³/h) = Air velocity (m/s) × Area (m²)

|  | Air velocity | Size of grille | Airflow volume |
| --- | --- | --- | --- |
| Grille 1 |  |  |  |
| Grille 2 |  |  |  |
| Grille 3 |  |  |  |

2. Smoke test

Release a small amount of smoke near:

An extract grille

A supply vent

Observe and record:

Direction of airflow

How quickly the smoke is drawn in

Whether air is being extracted from the space correctly

Illustrate your findings here:

An extract grille

A supply vent

3. Indoor air quality reading

Where possible, take measurements of:

CO₂ (ppm)

Humidity (%)

Temperature (°C)

These readings help you understand why ventilation is required.