| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 2: Gas Laws

Calculation Questions

1. Boyle’s Law (Constant Temperature)

Inside a compressor, refrigerant at 0.8 L and 150 kPa is compressed to 0.4 L.

What is the new pressure?

Answer:

_______________________________________

2. Charles’s Law (Constant Pressure)

A refrigerant gas in the evaporator is at 260 K with a volume of 2.0 L.

If it warms to 300 K, what will the new volume be?

Answer:

________________________________________

3. Gay Lussac’s Law (Constant Volume)

A sealed section of refrigerant is at 200 kPa at 290 K.

If the temperature rises to 330 K, what is the new pressure?

Answer:

________________________________________

4. Ideal Gas Law in HVAC

A refrigerant sample occupies 3.0 L at 320 K and 250 kPa.

Calculate the number of moles using:

PV = nRT,R = 8.314" J/mol

Answer:

________________________________________

5. Dalton’s Law

Air at sea level has 101 kPa pressure. If water vapour contributes 2.5 kPa, the pressure from dry air is:

a) 2.5 kPa

b) 50.5 kPa

c) 101 kPa

d) 98.5 kPa

Answer:

________________________________________

Application Questions

1. Explain why an AC system with low refrigerant may cause the evaporator coil to freeze. Reference at least one gas law in your explanation.

Answer:

________________________________________

2. A very hot day increases the pressure in the condenser. Explain how the combined gas law describes this behaviour.

Answer:

________________________________________

3. Why is it important for refrigerant to be in a high pressure, high temperature state before entering the condenser? Include gas law reasoning.

Answer:

________________________________________

4. A learner claims that “pressure and temperature in the compressor rise independently.”

Explain why this is incorrect using the appropriate gas law.

Answer:

________________________________________

5. If the expansion valve fails and does not reduce pressure properly, describe what will happen in the evaporator, referencing two gas laws.

Answer:

________________________________________