| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 5: Psychrometrics, Charts & Apps

Use a psychrometric chart or a reliable app. Take atmospheric pressure as 101.3 kPa unless stated.

1. Find RH and humidity ratio

At DBT = 26 °C and WBT = 18 °C: use a chart/app to determine:

(i) relative humidity (%),

(ii) humidity ratio (kg/kg dry air).

2. Properties at 22 °C, 50% RH

Find:

(i) humidity ratio,

(ii) specific volume (m³/kg),

(iii) moist air enthalpy (kJ/kg).

Then estimate sensible cooling from 22 °C to 18 °C at constant humidity ratio for 0.5 kg/s of dry air.

3. Air mixing

Mix 0.4 kg/s of air at 30 °C, 60% RH with 0.2 kg/s at 20 °C, 40% RH. Using a chart/app, estimate mixed-state enthalpy and DBT.

4. Refrigerant comparator

Using a refrigerant comparator app, find saturation temperature at 8 bar(g) for R‑410A and for R‑32. Record both values and comment which has the higher T_sat at the same pressure.

Extension

1) Using a chart/app, determine the **sensible heat ratio (SHR)** for cooling air from 26 °C, 50% RH to 18 °C supply air at ~95% RH at 0.6 kg/s dry air. Show how you obtained h and w values.

2) A room sensible load is 5.2 kW with a supply airflow of 0.90 m³/s. Estimate the supply air temperature difference (ΔT). Use ρ=1.2 kg/m³, c_p=1.005 kJ/kg·K.

3) Using a refrigerant comparator, at 35 °C find saturation pressure for R‑32 and R‑410A. Comment on which has the higher pressure at the same temperature.