| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 2: Heat Transfer

Air Conditioning Temperature Conversion Questions

1. Celsius → Kelvin

An AC technician measures the return air temperature in a building as 18°C.

What is this temperature in Kelvin?

Answer

________________________________________

2. Celsius → Kelvin

The outdoor condenser unit is operating at a coil temperature of 42°C.

Convert this temperature to Kelvin.

Answer

________________________________________

3. Kelvin → Celsius

A refrigerant line inside a commercial AC system is measured at 295 K.

What is the temperature in Celsius?

Answer

________________________________________

4. Kelvin → Celsius

During a system diagnostic, the evaporator coil temperature is recorded as 265 K.

What is the temperature in Celsius?

Answer

Heat Transfer Practice Problems

Problem 1 — Sensible Heat (Air Cooling)

Air enters an evaporator at 28°C and leaves at 18°C.

The airflow rate is 0.6 m³/s.

Calculate the cooling rate (kW).

________________________________________

Problem 2 — Sensible Heat (Heating Air)

Air is heated from 15°C to 30°C with an airflow of 0.4 m³/s.

Find the heating rate.

________________________________________

Problem 3 — Latent Heat (Dehumidification)

An evaporator removes 0.01 kg/s of water vapour from the air.

Latent heat of vaporisation L=2,500,000" J/kg".

________________________________________

Problem 4 — Conduction Heat Loss (Wall)

A cold room wall has:

Area: 12 m²

Thickness: 0.1 m

Thermal conductivity k=0.04" W/m\cdotpK"

Temperature difference across wall: 20°C

________________________________________

Problem 5 — Temperature Change Using Basic Formula

A refrigerant line carrying 2 kg/s of fluid is cooled by 5°C.

Specific heat c=1,400" J/kg\cdotp°C" .

________________________________________

Problem 6 — Mixed Sensible & Latent Cooling

An AC system cools air from 27°C to 20°C while also removing 0.004 kg/s of moisture.

Airflow is 0.5 m³/s.

What is the combined cooling?

________________________________________

Problem 7 — Combined Heat Gain

A small office has the following loads:

People: 1.5 kW

Lighting: 0.9 kW

Equipment: 1.2 kW

Solar gain: 2 kW

Calculate the total cooling load needed.

________________________________________

Problem 8 — Airflow Required

A classroom needs 6 kW of sensible cooling.

The expected temperature drop across the evaporator is 10°C.

Calculate the required airflow (m³/s).

Formula

Q ̇=1.2⋅V ̇⋅ΔT

________________________________________

Problem 9 — Condensate Production (From Latent Load)

An AC system provides 6 kW of latent cooling.

How much condensate is produced in litres per hour?

________________________________________

Problem 10 — Condensate Rate from Moisture Removal

The evaporator removes 0.005 kg of water every second.

How many litres per hour is this?