| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 1: Practical Air -Quality & Comfort Audit 

K1.4 Air Quality & Human Comfort

## Overview

Purpose: Apply knowledge of temperature, humidity, CO/CO₂ indicators, metabolism, pollutants, and maintenance in a realistic FE classroom setting.

Time: 10–15 minutes (plus 5 minutes for debrief).

Format: Pairs or small groups (2–3 learners).

## Learner Instructions

### 1. Measure or Estimate Environmental Conditions

Dry Bulb Temperature (DBT).

Relative Humidity (RH).

CO₂ Indicator (traffic‑light monitor if available, or qualitative cues such as stuffiness/drowsiness).

Air Movement (drafts, stagnant air, blocked vents).

### 2. Inspect for IAQ & Ventilation Issues

Blocked or dirty supply and extract vents.

Visible dust, staining, or condensation.

Damp smells, mustiness, or high humidity cues.

Noise or inactivity from mechanical ventilation.

Overheating or cold spots.

### 3. Identify Comfort Issues

Too hot / too cold?

Too humid / too dry?

Poor ventilation?

Pollutant indicators (e.g. VOC smells, dust levels)?

### 4. Recommend Improvements

Increase outdoor air flow.

Clean/replace filters.

Adjust thermostats and setpoints.

Improve airflow (clear obstructions at vents and intakes).

Address moisture ingress and ensure condensate drainage.

Request service for fans or dampers.

Learner Hand‑In Sheet – Practical Air‑Quality & Comfort Audit

K1.4 Air Quality & Human Comfort

## 1. Environmental Measurements / Estimates

Dry Bulb Temperature (DBT): __________________ °C

Relative Humidity (RH): __________________ %

CO₂ Indicator (monitor or cues): ______________________________________________

Air Movement (drafts, stagnant air, blocked vents): ______________________________

## 2. IAQ & Ventilation Observations

Condition of vents (clean/blocked/dusty): ______________________________________________

Signs of condensation/moisture: ______________________________________________

Odours (damp, VOCs, other): ______________________________________________

Ventilation system active? (Yes/No/Unsure): ______________________________________________

## 3. Comfort Issues Identified

Thermal comfort issues (too hot/cold): ______________________________________________

Humidity issues (too humid/dry): ______________________________________________

Airflow issues: ______________________________________________

Pollutant indicators: ______________________________________________

## 4. Recommended Improvements

Improvement 1 (with justification): ______________________________________________

_____________________________________________________________________________

Improvement 2 (with justification): ______________________________________________

_____________________________________________________________________________

## 5. Tutor Feedback

Comments: ___________________________________________________________________

Mark Awarded (out of 5): _________