| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 1: Data Types & SI Units (HVAC)

- Primary vs. Secondary data

State whether the data source is PRIMARY (you measured it) or SECONDARY (from another source).

a) Temperatures recorded by your own data logger on-site.

b) Manufacturer’s performance table from a datasheet.

c) Pressures read from your manometer during commissioning.

d) Values obtained from a refrigerant comparator app.

2. Match the measurement to the SI unit

Match each measurement to the correct SI unit (write the unit symbol).

a) Room air velocity __________

b) Static pressure in a duct __________

c) Heat transfer rate (power) __________

d) Energy used in 15 minutes __________

e) Density of moist air __________

f) Specific volume of air __________

3. Short conversions

i) 750 L = ______ m³

ii) 0.25 m³ = ______ L

iii) 1.2 kPa = ______ Pa

Extension

1) Convert 72 °F to Kelvin. Convert 3500 rpm to rad/s (SI).

2) Explain the difference between density and specific volume, giving SI units. If density is 1.18 kg/m³, find specific volume.