| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 2: SI Conversions & Dimensional Analysis

1. Convert the following

Use SI units and show all steps. Use prefixes (milli-, kilo-, etc.) correctly.

a) 2.8 L → m³

b) 1.35 bar → Pa

c) 650 kPa → mmH₂O

d) 12 m/s → km/h

e) 950 W → kW

f) 4.2 kJ → Wh

2. Prefix practice

a) 3.5 mm = ______ m

b) 2500 W = ______ kW

c) 0.85 m³/s = ______ L/s

3. Dimensional check

Mark ✓ if dimensionally consistent (SI base units), ✗ if not. Correct any that are wrong:

i) pressure = force / area (Pa = N/m²)

ii) energy = power × time (J = W·s)

iii) velocity = acceleration / time (m/s = m/s²·s)

iv) conductivity = W/m·K

4. Density & specific volume

a) A container holds 3.2 kg of air in a volume of 2.7 m³. Find density and specific volume.

b) A 15 L tank contains 13 kg of glycol solution. Estimate density in kg/m³.

Extension

1) Convert 250 Pa to:

(i) mmH₂O

(ii) bar

2) What pressure (kPa) corresponds to 3.5 m of water head? Use \( P = \rho g h \).