| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 1: Renewables ‘energy pathways’ card sort + mini-audit

Learning outcome

Identify key renewable energy sources and storage types.

Describe each pathway as: Source → Conversion → Output → Storage/Use.

Link at least two renewables to air-conditioning or building services use-cases.

Practical task A: Energy pathway card sort (20 min)

Your tutor will provide a set of ‘cards’ OR you can create your own using the lists below.

On A3 paper, build 4 complete pathways using at least: wind, solar PV, solar thermal, and one of hydro/tidal/biofuel/waste-to-energy.

For each pathway, include: (1) energy source, (2) conversion steps, (3) output (electricity/heat), (4) where storage can be added.

Card lists (use these if making your own)

Sources: wind / sunlight / river water / tides / biomass / municipal waste

Converters: rotor / turbine / generator / inverter / heat exchanger / boiler / engine / steam turbine

Outputs: electricity (AC/DC) / hot water / steam / district heat

Storage: battery / hot water cylinder / thermal store / ice/chilled water tank / pumped hydro

End uses (AC link): chiller / heat pump / absorption chiller / ventilation fans / pumps

Evidence: Paste or sketch your 4 pathways here

| Pathway name | Sketch/notes (source→conversion→output) | Storage point(s) | HVAC link |
| --- | --- | --- | --- |

Practical task B: Quick ‘building energy audit’ (15–20 min)

Look around your classroom/workshop area (or use a floor plan/photo if provided).

List 6 electrical loads and decide if each is: Essential / Non-essential / Shiftable.

For any ‘shiftable’ load, suggest when it could run to better match renewables (e.g., midday solar, windy night).

| Load (e.g., AC unit, PC) | Essential? | Shiftable? | When could it run? | Why? |
| --- | --- | --- | --- | --- |

Reflection (5 min)

What’s one thing you learned about how renewables connect to cooling?

What question do you still have?

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________