| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 2: Mechanical generation: turbine → generator mini-lab

Learning outcome

Explain how mechanical rotation becomes electrical energy (electromagnetic induction).

Collect simple measurements and interpret what they mean for renewable generators.

Relate the principle to wind, hydroelectric and tidal generation.

Safety

Keep fingers/hair clear of moving parts.

Do not exceed the rating of the device/load.

Use low-voltage lab equipment only (tutor will brief you).

Practical task A: Generate electricity (20–25 min)

Connect the hand generator to a small load (LED or USB load).

Crank at three different speeds: slow, medium, fast (use a stopwatch to estimate turns per 10 seconds).

Record voltage and current (or power if using a USB power meter).

Answer: What happens when speed increases? What does that suggest about wind/water speed and output?

| Trial | Turns in 10s | Voltage (V) | Current (A) | Power (W) |
| --- | --- | --- | --- | --- |

Analysis questions

1) Describe the energy conversions you observed (use arrows).

2) Where do losses occur? Give at least 3 examples.

3) Which renewable systems use this same turbine→generator idea? List at least 3.

4) If wind speed doubles, why might power rise a lot (hint: not linear)?

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

Challenge (optional)

Suggest one design change that could improve efficiency for (a) wind or (b) hydro/tidal.

Explain how power electronics can help connect variable-speed generators to the grid.

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________