| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 4: Storage practical: thermal store experiment + battery data

Learning outcome

Describe how thermal storage works and what affects heat losses.

Interpret battery charge/discharge data (state of charge, power, energy).

Compare storage types for an HVAC application.

Safety

Hot water can scald. Wear PPE if provided and follow tutor instructions.

Keep liquids away from electrical equipment.

Do not open any sealed battery packs. Use provided data only.

Practical task A: Thermal storage loss test (25–30 min)

Fill two identical bottles with the same hot water temperature (T0).

Insulate ONE bottle (wrap in cloth/insulation); leave the other uninsulated.

Measure temperature every 5 minutes for 20 minutes.

Plot a simple cooling curve (your tutor may provide graph paper).

| Time (min) | Insulated temp (°C) | Uninsulated temp (°C) | Notes |
| --- | --- | --- | --- |

Practical task B: Battery storage data (15 min)

Use the dataset provided (battery power vs. time or SOC vs. time).

Calculate total energy charged/discharged (kWh) over the period (your tutor will guide method).

Identify: peak power, average power, and one potential use in a building (e.g. peak shaving).

| Metric | Value | How you worked it out |
| --- | --- | --- |

Comparison: which storage is better for cooling? (10–15 min)

Complete the comparison table.

Then write a recommendation for a college with a chiller that peaks at 5pm.

| Feature | Battery storage | Thermal storage |
| --- | --- | --- |

Recommendation

_____________________________________________________________________________

_____________________________________________________________________________

_____________________________________________________________________________

_____________________________________________________________________________

_____________________________________________________________________________

_____________________________________________________________________________

_____________________________________________________________________________