| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 5: DSR/DSM practical: schedule an HVAC system to support the grid

Learning outcome

Explain DSR (short-term response) and DSM (broader management).

Use a tariff/carbon profile to plan load shifting.

Propose HVAC actions that maintain comfort and safety constraints.

Practical task A: Plan a daily schedule (25–30 min)

Your tutor will provide a 24-hour table of electricity price and/or carbon intensity.

Assume the building needs 40 kWh of cooling per day (chiller electricity).

You have an ice tank that can store 15 kWh of ‘cooling’.

Create a schedule that decides when to: (1) run chiller normally, (2) charge ice tank, (3) discharge ice tank.

Goal: reduce cost and/or carbon while staying within comfort limits.

| Time block | Price/Carbon (from tutor) | Action (run/charge/discharge) | kWh moved | Reason |
| --- | --- | --- | --- | --- |

Practical task B: Comfort & constraints check (10–15 min)

List at least 5 constraints that limit how much you can shift HVAC demand.

For each constraint, write one control strategy to stay safe (e.g., max temp band, min ventilation rate).

| Constraint | Why it matters | Control / mitigation |
| --- | --- | --- |

Output: short justification

Write a 150–200 word explanation of your schedule and how it supports the grid.

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________