| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 2: System Identification

K1.1 The function and operation of air conditioning systems

Your tutor will provide a building layout diagram and a schematic diagram for you to use.

- Given a building layout and equipment list, identify which areas are served by:

- DX systems

- Centralised chilled-water systems

- VRF/VRV systems

- Heat pumps

Provide justification for each answer.

- Examine the schematic diagram and label the major components of:

- A split DX system

- A chiller and chilled-water loop

- A VRF system including branch selector boxes (if present)

Tutor guidance

1. Identify systems by layout:

- Small rooms → DX

- Large open floors with ventilation → AHUs + central plant

- Multi-zone offices → VRF

- Buildings with high-efficiency requirement → Heat pump or chilled water.

Expect learners to give a reason for each answer.

2. Diagram labelling:
You would expect identification of compressor, evaporator, condenser, expansion valve, primary/secondary pumps, branch selector boxes, etc.