| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 1: Heat Pump Basics (Starter)

K1.10 Heat pumps | System maintenance

What you will do

Recall key components of a vapour-compression system.

Explain where heat is absorbed and where it is delivered in heating mode.

Use key terms correctly: evaporator, condenser, compressor, expansion device, superheat, subcool.

Instructions (10–15 minutes)

Work in pairs.

Answer the questions below in your own words.

Be ready to share one answer with the class.

Questions

1) Name the four main components of a vapour-compression heat pump cycle.

2) In heating mode, which heat exchanger gives useful heat to the building? Explain in one sentence.

3) In heating mode, where does the system absorb low-grade heat from the environment?

4) Define superheat (what does it compare?)

5) Define subcool (what does it compare?)

6) Why does a heat pump need electrical energy even though it “moves” heat?

Extension task (optional)

Explain what “temperature lift” means and why a bigger temperature lift usually reduces COP.

Tutor guidance

1) Name the four main components of a vapour‑compression heat pump cycle.

- Evaporator

- Compressor

- Condenser

- Expansion device

2) In heating mode, which heat exchanger gives useful heat to the building?

The condenser gives useful heat to the building because this is where the refrigerant releases the heat it has absorbed and compressed. (vapour to liquid at high pressure)

3) In heating mode, where does the system absorb low‑grade heat from the environment?

At the evaporator, which extracts heat from the outside air, ground, or water source depending on the heat pump type.

4) Define superheat (what does it compare?)

Superheat is the temperature of the refrigerant vapour above its saturation (boiling) temperature at a given pressure.

5) Define subcool (what does it compare?)

Subcool is the temperature of the refrigerant liquid below its saturation (condensing) temperature at a given pressure.

6) Why does a heat pump need electrical energy even though it “moves” heat?

A heat pump needs electrical energy to drive the compressor, which raises the refrigerant pressure and temperature so heat can be transferred into the building. (cold source to a warmer source)

Extension task (optional)

Temperature lift is the difference between the heat source temperature (evaporator) and the heat delivery temperature (condenser). A bigger temperature lift requires the compressor to do more work, increasing electrical input and lowering COP.