| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 3: Unexpected Noise: Matching, Locating, and First Checks

A) Noise matching (card sort)

Match each noise to a likely cause and write a safe first check. Use your own words.

Complete the table

| Noise description | Likely component (compressor/fan/bearing/panel) | Likely cause (brief) | First safe check |
| --- | --- | --- | --- |
| Rattle / buzzing |  |  |  |
| Squeal |  |  |  |
| Grinding / scraping |  |  |  |
| Knock / clunk |  |  |  |
| High-pitched whine |  |  |  |

B) Locating a noise (practical)

Your tutor will demonstrate a “noise source” using a safe prop/audio. Write down a safe method to locate the source without touching moving parts.

1. Describe a safe way to move around the unit to locate the noise source.

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

2. What operating changes can help you diagnose (e.g., change fan speed) and why?

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

3. When would you stop and escalate immediately?

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Tutor guidance

A) Noise matching – model answers

Completed table (model)

| Noise | Likely component | Likely cause (brief) | First safe check |
| --- | --- | --- | --- |
| Rattle / buzzing | Panel / pipework | Loose panel, missing screw, pipe contact, cable tray vibration | Press panel lightly (if safe) to see change; inspect fixings and rub marks |
| Squeal | Bearing / belt-driven fan | Worn/dry bearing or belt slip (if belt-driven) | Listen near motor housing; check for heat; inspect belt tension/alignment |
| Grinding / scraping | Fan | Blade contacting guard/casing; debris; bearing failure | Visual check clearances/guard; stop and escalate if contact suspected |
| Knock / clunk | Compressor / mounts | Loose mounts, internal compressor issue, loose component | Inspect mounts/fixings; record when occurs; escalate if persistent/metallic |
| High-pitched whine | Fan/motor control | Inverter/high speed resonance or normal control noise | Compare to baseline; vary speed if permitted; check for resonance/loose panels |

B) Locating a noise – model responses

1. Keep a safe distance from moving parts and move slowly around the unit, listening for where the noise is loudest. Use “point and describe” rather than touching. Do not lean over guards.

2. Change fan speed or operating mode (if permitted and safe). If noise changes with speed, it suggests fan/bearing/resonance. If it changes with compressor load, suspect compressor/mounts.

3. Stop/escalate immediately if: grinding/scraping contact, sudden loud metallic noise, smoke/burning smell, arcing/buzzing from electrical parts, or severe vibration.