| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 6: Fault Clues Case Studies (Decision Making & Communication)

Case Study A

Noise: scraping increases with fan speed. Sight: fan guard appears bent. Touch: vibration on outdoor panel. Smell: none.

1. Most likely component causing the noise?

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

2. Your first three SAFE checks (in order).

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

3. Monitor/fix/escalate? Explain your decision.

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Case Study B

Smell: hot electrical/burning near indoor unit. Sight: discoloured cable insulation near terminal area. Hearing: intermittent buzzing.

1. What should you do immediately?

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

2. What evidence would you record?

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

3. Why is this a high-risk situation?

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Case Study C

Hearing: squeal from fan area. Touch: motor housing feels hotter than usual (do not touch terminals). Sight: no obvious debris.

1. What is a likely cause?

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

2. What change could you make (safely) to help diagnose?

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

3. When would you escalate?

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Practical communication task (role-play)

One learner is the engineer, one is the customer, one is the supervisor. Explain the finding in simple terms and state what happens next.

1. Write a 3–4 sentence “customer friendly” explanation for one case study above.

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

2. Write a 3–4 sentence “technical report” summary for the supervisor (include what/where/when/evidence/action).

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Tutor guidance

Case Study A – model answers

1. Most likely component: fan/guard area (blade contacting guard/casing) or bent guard causing contact.

2. First three safe checks: (1) stop/isolate if safe and per procedure; (2) visual inspection of guard/clearances and fixings; (3) check for debris and verify whether noise changes with speed setting.

3. Decision: escalate if contact/grinding confirmed or if noise is severe/sudden; otherwise fix guard alignment/tighten fixings by competent person and re-check.

Case Study B – model answers

1. Immediate action: stop the unit and isolate power following procedure; escalate to competent person.

2. Evidence: description of smell, buzzing, visible discolouration; photo; time/date; operating mode; location (which terminal area).

3. High risk: overheating or loose electrical termination can lead to arcing, fire, or electric shock.

Case Study C – model answers

1. Likely cause: worn/dry bearing in fan motor; could also be belt slip if belt-driven.

2. Safe diagnostic change: reduce/increase fan speed (if permitted) and see if squeal changes; compare to baseline; check for temperature rise at motor housing with IR/thermometer.

3. Escalate if squeal becomes grinding, motor overheats, smoke/burning smell appears, or vibration increases significantly.

Practical communication role-play – model scripts

Customer-friendly example (3–4 sentences):

“We’ve noticed an unusual sound from the outdoor fan, and it changes when the fan speed changes. That suggests something may be rubbing or out of line. For safety we will stop the unit, and have it checked and secured before it’s used. We’ll record what we found and update you with the next steps.”

Supervisor technical summary example (3–4 sentences):

“During inspection in cooling mode at steady-state, scraping noise increased with fan speed. Visual check suggests bent/loose fan guard; vibration felt on outdoor panel. Action: stopped unit and isolated per procedure; recorded photos and operating conditions. Recommend competent engineer to inspect guard clearance, fan alignment and fixings, then re-test.”