| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 3 – Demonstrate Removal of Stale or Harmful Air

K1.2 Air conditioning and ventilation in a modern economy

Objective:

Learners will see how ventilation systems remove polluted air and replace it with fresh air.

Tutor guidance

Evidence learners produce:

✔ Before/after readings
✔ Annotated airflow diagram
✔ Explanation of fresh-air requirements

Question 1: Effects of blocking the fresh‑air intake

Observed changes when the fresh-air intake is partially blocked:

- CO₂ levels
CO₂ levels rise because the same indoor air keeps being recirculated and fresh outdoor air is not entering the system.

- Humidity
Humidity increases as moisture from people, breathing, and activities (e.g., cleaning, equipment operation) cannot escape.

- Smell / Stale feeling
The room starts feeling stuffy, warm, and uncomfortable. Odours linger because there is no dilution of pollutants.

Explanation – Why a fresh-air supply matters:
A fresh-air supply is essential for maintaining good indoor air quality. It removes CO₂, excess humidity, smells, and airborne pollutants while bringing in clean air. This helps prevent headaches, tiredness, and poor concentration, and ensures the ventilation system maintains a healthy indoor environment.

Question 2: Opening the fresh‑air intake

Improvements after opening the fresh-air intake:

- CO₂ levels
CO₂ levels drop as polluted indoor air is replaced with oxygen-rich outdoor air.

- Humidity
Humidity reduces because moisture-laden air is extracted and replaced with drier fresh air.

- Overall air quality
The room feels fresher, cooler, and cleaner. Smells disappear more quickly, and comfort improves.

Comparison (Before vs After):

| Condition | Before (Intake Blocked) | After (Intake Open) |
| --- | --- | --- |
| CO₂ Levels | Rising | Lower, more stable |
| Humidity | Increasing | Reduced |
| Smell | Stale, stuffy | Fresh, cleaner |
| Comfort | Poor | Improved |

Opening the intake restores proper ventilation and maintains healthy, breathable air.

Question 3: Path of stale air through the ventilation system

Stale air route:
Learners should trace the path by following these components in order:

- Extract grille – where stale air is pulled out of the room.

- Ductwork – carries the extracted air through the system.

- Extract/ventilation fan – provides suction to move the air along the ducts.

- Outside discharge point – stale air is expelled safely outdoors.

Diagram

Suggested route to show the complete journey of stale indoor air as it is removed from the building by the ventilation system.