| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 2: Gas Laws & Pressure Measurement

Questions

Q1. Convert 2.5 bar gauge to bar abs, then to kPa, mmHg, torr.

Show all working and give answers with correct units.

Given:

1 bar = 100 kPa

1 atm = 1.013 bar = 760 mmHg = 760 torr

Q2. Nitrogen cylinder: 180 bar abs, 12 L → 90 bar abs (isothermal). Find new volume.

Show all working and give answers with correct units.

Q3. Refrigerant vapour 0.85 m³ at 15 °C → 35 °C at constant pressure. Find new volume (use K).

Show all working and give answers with correct units.

Q4. Combined gas law: 1.8 bar abs & 12 L at 10 °C → 9 L at 40 °C. Find pressure.

Show all working and give answers with correct units.

Q5. Dalton: dry air 0.75 bar + water vapour 0.03 bar. Total pressure and % contribution of vapour?

Show all working and give answers with correct units.

Q6. Pressure test with nitrogen: 28 bar g at 18 °C. Next morning 12 °C, no leaks. Expected gauge pressure?

Show all working and give answers with correct units.

Tutor guidance

Model answers (one of many possible solution methods).

Q1 – Answer

3.513 bar abs; 351.3 kPa; ~2,636 mmHg/torr

Q2 – Answer

24 L

Q3 – Answer

0.91 m³ (approx)

Q4 – Answer

2.66 bar abs (~1.65 bar g)

Q5 – Answer

0.78 bar abs; ~3.85% water vapour

Q6 – Answer

≈27.4 bar g