| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 2: Gas Laws

Calculation Questions

1. Boyle’s Law (Constant Temperature)

Inside a compressor, refrigerant at 0.8 L and 150 kPa is compressed to 0.4 L.

What is the new pressure?

Answer

Given:

P_1=150" kPa"

V_1=0.8" L"

V_2=0.4" L"

Formula:

P_1 V_1=P_2 V_2

Solve for P_2:

150×0.8=P_2×0.4

120=0.4P_2

P_2=120/0.4=300" kPa"

Final Answer: 300 kPa

2. Charles’s Law (Constant Pressure)

A refrigerant gas in the evaporator is at 260 K with a volume of 2.0 L.

If it warms to 300 K, what will the new volume be?

Answer

Given:

V_1=2.0" L"

T_1=260" K"

T_2=300" K"

Formula:

V_1/T_1 =V_2/T_2

Solve for V_2:

V_2=V_1×T_2/T_1

V_2=2.0×300/260

V_2=2.0×1.1538=2.31" L"

Final Answer: 2.31 L

3. Gay Lussac’s Law (Constant Volume)

A sealed section of refrigerant is at 200 kPa at 290 K.

If the temperature rises to 330 K, what is the new pressure?

Answer

Given:

P_1=200" kPa"

T_1=290" K"

T_2=330" K"

Formula:

P_1/T_1 =P_2/T_2

Solve for P_2:

P_2=P_1×T_2/T_1

P_2=200×330/290

P_2=200×1.1379=227.6" kPa"

Final Answer: 228 kPa (rounded)

4. Ideal Gas Law in HVAC

A refrigerant sample occupies 3.0 L at 320 K and 250 kPa.

Calculate the number of moles using:

PV = nRT,R = 8.314" J/mol

Answer

Given:

P=250" kPa"=250,000" Pa"

V=3.0" L"=0.003〖" m" 〗^3

T=320" K"

R=8.314

Formula:

PV=nRT

Solve for n:

n=PV/RT

n=(250000×0.003)/(8.314×320)

n=750/2660.48

n=0.282" mol"

Final Answer: 0.28 mol

5. Dalton’s Law

Air at sea level has 101 kPa pressure. If water vapour contributes 2.5 kPa, the pressure from dry air is:

a) 2.5 kPa

b) 50.5 kPa

c) 101 kPa

d) 98.5 kPa

Suggested answer

P_"dry air" =P_"total" -P_("H" _2 "O" )=101-2.5=98.5" kPa"

Application Questions

1. Explain why an AC system with low refrigerant may cause the evaporator coil to freeze. Reference at least one gas law in your explanation.

Suggested answer

Low charge → fewer moles (Avogadro’s Law) → lower pressure in evaporator.

Lower pressure → lower temperature (Gay Lussac’s Law).

Temperature can drop below 0°C, freezing moisture on the coil.

2. A very hot day increases the pressure in the condenser. Explain how the combined gas law describes this behaviour.

Suggested answer

Hot outdoor air raises condenser coil temperature.

According to Gay Lussac’s Law:

P∝T

Higher temperature → higher pressure.

The Combined Gas Law also applies since P, V, T all shift due to heat load.

3. Why is it important for refrigerant to be in a high pressure, high temperature state before entering the condenser? Include gas law reasoning.

Suggested answer

A condenser needs refrigerant hotter than outside air to reject heat.

Compression raises pressure → raises temperature (Gay Lussac).

This allows the condenser to dump heat and condense the refrigerant to liquid.

4. A learner claims that “pressure and temperature in the compressor rise independently.”

Explain why this is incorrect using the appropriate gas law.

Suggested answer

Gay Lussac’s Law:

P∝T

In a sealed compressor, pressure and temperature rise together, not independently.

Compressing gas forces particles closer → increases collisions → increases temperature.

5. If the expansion valve fails and does not reduce pressure properly, describe what will happen in the evaporator, referencing two gas laws.

Suggested answer

If the valve doesn’t reduce pressure:

Gay Lussac’s Law:

Pressure stays high → temperature stays high → evaporator won’t get cold.

Charles’s Law:

Volume doesn’t expand → no temperature drop.

Evaporator fails to absorb heat → poor cooling.