| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 1: Renewables ‘energy pathways’ card sort + mini-audit

Learning outcome

Identify key renewable energy sources and storage types.

Describe each pathway as: Source → Conversion → Output → Storage/Use.

Link at least two renewables to air-conditioning or building services use-cases.

Practical task A: Energy pathway card sort (20 min)

Your tutor will provide a set of ‘cards’ OR you can create your own using the lists below.

On A3 paper, build 4 complete pathways using at least: wind, solar PV, solar thermal, and one of hydro/tidal/biofuel/waste-to-energy.

For each pathway, include: (1) energy source, (2) conversion steps, (3) output (electricity/heat), (4) where storage can be added.

Card lists (use these if making your own)

Sources: wind / sunlight / river water / tides / biomass / municipal waste

Converters: rotor / turbine / generator / inverter / heat exchanger / boiler / engine / steam turbine

Outputs: electricity (AC/DC) / hot water / steam / district heat

Storage: battery / hot water cylinder / thermal store / ice/chilled water tank / pumped hydro

End uses (AC link): chiller / heat pump / absorption chiller / ventilation fans / pumps

Evidence: Paste or sketch your 4 pathways here

| Pathway name | Sketch/notes (source→conversion→output) | Storage point(s) | HVAC link |
| --- | --- | --- | --- |

Practical task B: Quick ‘building energy audit’ (15–20 min)

Look around your classroom/workshop area (or use a floor plan/photo if provided).

List 6 electrical loads and decide if each is: Essential / Non-essential / Shiftable.

For any ‘shiftable’ load, suggest when it could run to better match renewables (e.g. midday solar, windy night).

| Load (e.g. AC unit, PC) | Essential? | Shiftable? | When could it run? | Why? |
| --- | --- | --- | --- | --- |

Reflection (5 min)

What’s one thing you learned about how renewables connect to cooling?

What question do you still have?

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Tutor guidance

Use as a model answer + marking guidance. Accept equivalent correct wording.

Model pathways (examples)

• Wind → rotor/blades → shaft/gearbox (or direct drive) → generator → power electronics → AC electricity → (optional) battery/thermal store → HVAC loads (chiller, fans).

• Solar PV → PV modules → DC electricity → inverter → AC electricity → (optional) battery → HVAC/IT loads; surplus export.

• Solar thermal → collector → heat-transfer fluid loop + pump → heat exchanger in cylinder/thermal store → hot water/heat → (optional) absorption cooling or DHW.

• Hydroelectric → water head/flow → turbine (impulse/reaction) → generator → AC electricity → grid; (pumped hydro adds storage).

• Tidal stream → tidal current → turbine → generator → AC electricity → grid (predictable output profile).

• Biofuel CHP → fuel combustion → heat → engine/turbine → generator → electricity + recovered heat (CHP) → heat for DHW/absorption cooling.

• Waste-to-energy (incineration) → combustion → boiler → steam → steam turbine → generator → electricity + heat recovery; residues/ash managed.

• Anaerobic digestion → biogas → engine/turbine → generator → electricity + heat; digestate by-product.

Common misconceptions to correct

• Storage does not create energy; it shifts energy in time and has losses.

• Solar thermal produces heat, not electricity (unless part of a steam/CSP system at utility scale).

• “Renewable” does not always mean “zero carbon” (biofuels/waste-to-energy depend on lifecycle emissions and feedstock).

Mini-audit: example classifications

• Essential: emergency lighting, server/IT room cooling (if required), safety systems.

• Non-essential: decorative lighting, some plug-in chargers left on.

• Shiftable: dishwashers, battery charging, chilled-water/ice charging, some IT tasks/scheduled maintenance, pre-cooling.

• Good “when to run”: midday (high solar), overnight (often lower demand; may align with wind), or when price/carbon is low depending on signals.

Marking guidance (suggested /10)

Criterion	What good looks like	Marks

| Criterion | What good looks like | Marks |
| --- | --- | --- |
| 4 pathways correct | Each includes source → conversion → output; at least one includes storage correctly. | 4 |
| HVAC link | At least two clear links to cooling/building services, explained briefly. | 2 |
| Audit quality | 6 loads listed with sensible essential/shiftable reasoning and timing. | 3 |
| Reflection | One clear learning point + a question. | 1 |