| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 2: Mechanical generation: turbine → generator mini-lab

Learning outcome

Explain how mechanical rotation becomes electrical energy (electromagnetic induction).

Collect simple measurements and interpret what they mean for renewable generators.

Relate the principle to wind, hydroelectric and tidal generation.

Safety

Keep fingers/hair clear of moving parts.

Do not exceed the rating of the device/load.

Use low-voltage lab equipment only (tutor will brief you).

Practical task A: Generate electricity (20–25 min)

Connect the hand generator to a small load (LED or USB load).

Crank at three different speeds: slow, medium, fast (use a stopwatch to estimate turns per 10 seconds).

Record voltage and current (or power if using a USB power meter).

Answer: What happens when speed increases? What does that suggest about wind/water speed and output?

| Trial | Turns in 10s | Voltage (V) | Current (A) | Power (W) |
| --- | --- | --- | --- | --- |

Analysis questions

1) Describe the energy conversions you observed (use arrows).

2) Where do losses occur? Give at least 3 examples.

3) Which renewable systems use this same turbine→generator idea? List at least 3.

4) If wind speed doubles, why might power rise a lot (hint: not linear)?

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

Challenge (optional)

Suggest one design change that could improve efficiency for (a) wind or (b) hydro/tidal.

Explain how power electronics can help connect variable-speed generators to the grid.

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

Tutor guidance

Use as a model answer + marking guidance. Accept equivalent correct wording.

Expected observations

As cranking speed increases, measured voltage typically increases; current/power also increases for a given load (up to device limits).

Under heavier electrical load, cranking feels harder because mechanical input power must increase to supply electrical output + losses.

Output is variable and depends on speed and load — similar to variable wind/water speeds and generator control.

Model answers to analysis questions

1) Energy conversions: chemical energy (muscles) → mechanical rotation → electrical energy → light/heat/motion in the load.

2) Losses: friction in bearings/gears; electrical resistance heating; magnetic/eddy current losses; power electronics losses; air resistance.

3) Same principle: wind turbines, hydroelectric turbines, tidal stream turbines, steam turbines in thermal plants (biofuel/waste-to-energy).

4) Wind power increases strongly with wind speed (concept: proportional to v³), so doubling speed can increase available power many times (idealised).

Extension guidance

Efficiency improvements: better blade aerodynamics, reduced mechanical friction, direct drive, improved generator design, better control (pitch/yaw or wicket gates).

Power electronics: rectify/convert variable frequency/voltage to grid-compatible output; enable variable-speed operation while maintaining 50 Hz grid connection.

Marking guidance (suggested /12)

| Criterion | What good looks like | Marks |
| --- | --- | --- |
| Data table completed | 3+ trials with plausible readings and units. | 3 |
| Trend explained | Correct explanation: higher speed → higher electrical output; load affects torque. | 3 |
| Losses | At least 3 valid loss points. | 2 |
| Renewable links | At least 3 correct systems identified. | 2 |
| Non-linear wind point | Shows understanding that output can rise rapidly with speed. | 2 |