| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 3: Solar PV vs solar thermal: build a system diagram + use-case

Learning outcome

Distinguish between solar PV (electricity) and solar thermal (heat).

Draw accurate system diagrams and label key components.

Choose the best technology for two building services scenarios.

Practical task A: Diagram build (25 min)

Draw TWO block diagrams: (1) Solar PV system and (2) Solar thermal system.

Include at least: PV modules/collector, cabling/pipework, inverter/pump, storage (battery/cylinder), and end use.

Add arrows showing energy flow (DC/AC for PV; heat-transfer fluid loop for thermal).

| System | Your labelled diagram (space for sketch) | Key components list |
| --- | --- | --- |

Practical task B: Scenario choice (15–20 min)

Scenario 1: A college wants to reduce daytime electricity import for cooling and IT loads.

Scenario 2: A sports centre wants to reduce gas/electricity used for showers/hot water.

For each scenario, choose PV or solar thermal (or both) and justify your decision.

Include how you would use storage to improve usefulness.

| Scenario | Your choice (PV/thermal/both) | Justification | Storage plan |
| --- | --- | --- | --- |

Mini-calculation (optional)

A PV system produces 10 kWh between 10:00–16:00. The building can use 6 kWh during that time.

a) How much surplus is available for storage/export?

b) If a battery is 90% efficient round-trip, how much usable energy could you get later?

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Tutor guidance

Answers

Diagram checklist – PV

- PV modules → DC cabling → inverter (DC→AC) → distribution board/loads/grid.

- Optional: battery (DC-coupled or AC-coupled), charge controller/BMS, meter/export.

- Correct labels: DC before inverter, AC after inverter.

Diagram checklist – solar thermal

- Solar collector (flat plate or evacuated tubes) → heat-transfer fluid loop → pump/controller → heat exchanger coil in cylinder/thermal store → hot water output.

- Return line back to collector; antifreeze mix for frost protection.

- Backup heater/boiler often required (especially in winter).

Scenario model answers

- Scenario 1 (college reducing daytime import for cooling/IT): PV is best match (electricity for chillers/IT). Add battery or pre-cooling/thermal storage to extend benefit to late afternoon.

- Scenario 2 (sports centre hot water): solar thermal is usually most direct/efficient for DHW; PV can help but thermal is designed for hot water production

- Storage: PV → batteries/thermal (ice) for time-shifting; solar thermal → hot water cylinder/thermal store.

Mini-calculation answers

- Surplus = 10 kWh − 6 kWh = 4 kWh.

- Usable after battery (90% round-trip) ≈ 4 × 0.90 = 3.6 kWh.

Marking guidance (suggested /12)

| Criterion | What good looks like | Marks |
| --- | --- | --- |
| PV diagram accuracy | Correct components and DC/AC labelling. | 3 |
| Thermal diagram accuracy | Correct closed-loop and store/backup. | 3 |
| Scenario 1 justification | Chooses PV (or PV+thermal) with sound reasoning linked to loads. | 2 |
| Scenario 2 justification | Chooses solar thermal for DHW with sound reasoning. | 2 |
| Storage discussion | Clear plan for storage with purpose. | 1 |
| Mini-calc | Correct arithmetic and units. | 1 |