| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 4: Storage practical: thermal store experiment + battery data

Learning outcome

Describe how thermal storage works and what affects heat losses.

Interpret battery charge/discharge data (state of charge, power, energy).

Compare storage types for an HVAC application.

Safety

Hot water can scald. Wear PPE if provided and follow tutor instructions.

Keep liquids away from electrical equipment.

Do not open any sealed battery packs. Use provided data only.

Practical task A: Thermal storage loss test (25–30 min)

Fill two identical bottles with the same hot water temperature (T0).

Insulate ONE bottle (wrap in cloth/insulation); leave the other uninsulated.

Measure temperature every 5 minutes for 20 minutes.

Plot a simple cooling curve (your tutor may provide graph paper).

| Time (min) | Insulated temp (°C) | Uninsulated temp (°C) | Notes |
| --- | --- | --- | --- |

Practical task B: Battery storage data (15 min)

Use the dataset provided (battery power vs. time or SOC vs. time).

Calculate total energy charged/discharged (kWh) over the period (your tutor will guide method).

Identify: peak power, average power, and one potential use in a building (e.g. peak shaving).

| Metric | Value | How you worked it out |
| --- | --- | --- |

Comparison: which storage is better for cooling? (10–15 min)

Complete the comparison table.

Then write a recommendation for a college with a chiller that peaks at 5pm.

| Feature | Battery storage | Thermal storage |
| --- | --- | --- |

Recommendation

_____________________________________________________________________________

_____________________________________________________________________________

_____________________________________________________________________________

_____________________________________________________________________________

_____________________________________________________________________________

_____________________________________________________________________________

_____________________________________________________________________________

Tutor guidance

Use as a model answer + marking guidance. Accept equivalent correct wording.

Thermal loss test – expected result

Insulated bottle should cool more slowly (smaller temperature drop over the test period).

Uninsulated bottle shows faster cooling due to higher heat transfer to surroundings.

Good discussion points: surface area, insulation thickness, convection, radiation, starting temperature, measurement error.

Battery data task – how to mark

If given power vs. time, energy (kWh) ≈ sum of (power in kW × time in hours) for each interval.

Peak power = highest kW value; average power = total energy ÷ total time.

Acceptable use-case interpretations: peak shaving, backup, PV time-shifting, frequency response (concept).

Note: student worksheet uses a dataset provided by tutor. Mark the method and working even if numbers differ.

Comparison & recommendation – model points

Battery: stores electricity; flexible uses; higher cost per kWh; needs inverter/BMS; limited cycle life; can serve many loads.

Thermal: stores heat/cold; usually cheaper per kWh for HVAC; losses over time; requires space/tanks; only serves thermal loads.

For a 5pm cooling peak: thermal storage (ice/chilled water) is often very effective; batteries also help for short peaks or critical loads.

Marking guidance (suggested /12)

| Criterion | What good looks like | Marks |
| --- | --- | --- |
| Thermal dataset | Temperature readings logged at correct intervals; clear comparison. | 3 |
| Cooling curve interpretation | States insulated cools slower + explains why. | 3 |
| Battery calculations method | Correct energy method and clear working. | 3 |
| Comparison & recommendation | Balanced comparison + justified choice for 5pm peak. | 3 |