| Learner name: |  | Class: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

Task 6: Biofuels & waste-to-energy: sustainability evaluation mini-project

Learning outcome

Describe how biofuels and waste-to-energy generate electricity (thermal → mechanical → electrical).

Evaluate sustainability using simple criteria (feedstock, emissions, waste hierarchy).

Make a recommendation for a local/community context.

Practical task A: Process mapping (15–20 min)

Choose ONE: (1) Biofuel CHP plant or (2) Waste-to-energy incineration or (3) Anaerobic digestion to biogas.

Create a flow diagram from input → processing → energy outputs (electricity/heat) → residues.

Label where mechanical generation happens and where emissions controls might be needed.

| Chosen system | Flow diagram (space for sketch) | Key outputs + residues |
| --- | --- | --- |

Practical task B: Sustainability scorecard (20 min)

Score each criterion 1–5 (1 = poor, 5 = excellent).

Add a short note explaining your score.

Then calculate the total and decide if you would recommend it.

| Criterion | Score (1–5) | Evidence / notes |
| --- | --- | --- |
| Feedstock sustainability (source, renewability) |  |  |
| Lifecycle emissions (including transport) |  |  |
| Air quality / pollutants control |  |  |
| Impact on land use / biodiversity |  |  |
| Reliability/dispatchability (can it run when needed?) |  |  |
| Local community impacts (noise, traffic, acceptance) |  |  |
| Circular economy benefits (reduces waste/landfill?) |  |  |
| Cost/feasibility for the location |  |  |
| Total score |  |  |

Recommendation (10–15 min)

Write a recommendation: Should your chosen system be used to supply energy for a community/building?

Include: 2 benefits, 2 risks, and 1 improvement to make it more sustainable.

Link it back to building services (heat recovery, absorption cooling, or grid support).

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

________________________________________________________________________________

Tutor guidance

Practical task A: Process mapping

Step 1 – Help learners outline the basic flow

- Use a simple chain:

Feedstock → Processing → Thermal energy → Mechanical shaft power → Electricity → Heat recovery → Residues/emissions

Step 2 – Key teaching points

- Emphasise thermal → mechanical → electrical conversion (boiling water → turbine → generator).

- Identify where emissions controls exist (filters, scrubbers, catalytic converters).

- Identify energy outputs: electricity and, if CHP, usable heat.

- Reinforce that biogas or combustion heat is the driver in all systems.

Model guidance for each option

(1) Biofuel CHP

- Input: Wood chips, pellets, or liquid biofuel

- Processing: Combustion in boiler → steam → turbine

- Outputs: Electricity, heat for district heating

- Residues: Ash (can fertilise soil)

(2) Waste‑to‑Energy Incineration

- Input: Municipal solid waste

- Processing: High‑temperature incineration → steam → turbine

- Outputs: Electricity + heat

- Controls: Flue gas cleaning (scrubbers, bag filters)

- Residues: Bottom ash, fly ash requiring safe disposal

(3) Anaerobic Digestion → Biogas

- Input: Food waste, agricultural slurry

- Processing: Microbial digestion → biogas → engine → generator

- Outputs: Electricity, heat

- Residues: Digestate (nutrient‑rich fertiliser)

Practical Task B – Sustainability

Tutor Instructions

Step 1 – Explain each criterion clearly

- Feedstock sustainability – Is the material renewable or waste‑based?

- Lifecycle emissions – Includes transport, processing, combustion/methane leaks.

- Air quality impacts – NOx, particulates, controls installed.

- Land use / biodiversity – Impact of growing biomass or siting facilities.

- Reliability – Does the source run 24/7? (e.g. AD and WtE do; some biofuels depend on supply).

- Community impact – Traffic, odour, noise, visual impact.

- Circular economy – Does it reduce landfill? Recover nutrients?

- Cost/feasibility – Is it realistic for the local area?

Step 2 – Encourage evidence‑based scoring

Examples:

- “I scored 4 because…”

- “Evidence: reduces landfill by 30%…”

- “Local waste is available, so feedstock supply is reliable.”

Step 3 – Help learners total the score

A rough guide for tutors (not shown to learners unless you want to):

- 32–40 = Strongly recommend

- 24–31 = Recommend with minor concerns

- 16–23 = Significant risks – recommend only with improvements

- <16 = Not suitable

Recommendation Section

Tutor Guidance

Benefits examples:

- Uses local waste and reduces landfill

- Provides steady baseload power

- Heat can support district heating or absorption chillers

Risks examples

- Air quality concerns

- High capital cost

- Community resistance due to traffic/odour

Improvement examples

- Adding advanced emissions controls

- Increasing recycling before incineration

- Using digestate responsibly to avoid nutrient runoff

Building services link

Prompts you may give learners:

- How could waste heat support buildings on-site?

- Could CHPs stabilise local grids?

- Could surplus heat drive an absorption chiller?